Problem 1

- Model (the flow out ‘F_out’ is assumed to be a state in the model)

dX1dt=KpA u-1AX2

dX2dt=0

h=X1+v                v = measurement noise

A, B, C and D of the linear model

A=0-1A00  ; B=KpA0  ;  C=10 ;  D=0

Kalman Filter parameters

Q=0.01000.0001;R=0.000001 ;Pini=1001

The Kalman filter algorithm is implemented in matlab and the code can be found in Appendix, the code is also saved as a file Problem_1.m.  The simulator is run using the values of Q and R presented in the question. The code first asks to choose between the options of a. Change in set point and b. Change in ‘Flow_out’ and provides the values of Kalman Gain and plots the graph between the estimated value and the real value of level and the Flow out. The following Kalman gain is obtained from the values of Q and R provided in the question.

K = [0.9999 -0.0951]

Figure 1. Kalman Filter estimation of F_out and tank level for change in Set point

Figure 2. Kalman Filter estimation of F_out and tank level for change Outflow from the tank

The measurement noise is increased to 0.005 and the simulator is run with a different value of R. The comparison shown in the figure below demonstrates the influence of R is estimating the F_out.

Figure 3. Kalman Filter estimation of F_out for a. R=0.000001 and b. R=1

Increasing the value of R provides a less noisy estimation of F_out but it the estimator demonstrates a slower convergence to the real value.

Figure 4, 5 and 6 shows the influence of changing the values of Q matrix in estimating F_out. Increasing the value of Q (1,1) makse the estimator converge slower to the real value but it could provide less noisier estimate. Lowering the value of Q (2,2) make the estimator converge slower to the real value but it could provide less noisier estimate.

Figure 4. Kalman Filter estimation of F_out for a. Q(1)=0.01 b. Q(1)=0.1  and a. Q(1)=1

Figure 5. Kalman Filter estimation of F_out for a. Q(2)=0.01 b. Q(2)=0.001  and a. Q(2)=0.0001

Figure 6. Kalman Filter estimation of F_out for a. Q(2)=0.01 b. Q(2)=0.001  and a. Q(1)=0.0001

- Parameter Estimation using Kalman Filter

In this case, the parameter Kv defined as a state X2

dX1dt=KpA u-1Af(z)X2ρgX1G

dX2dt=0

For the case of simplicity f(z) is assumed to be a linear function in z.

f(z)=k*z

The model is implemented in a symbolic form, which can be linearized to obtain the A, B, C, D matrix that can be used to calculate the Kalman Gain and to estimate the state X2 (or the parameter Kv)

The code for the model is in the appendix, it can also be found with the name Problem_1b.m.

The model is non-linear because it cannot be represented directly in the form

dXdt=A x+Bu

y=C x+Du

Problem 2

The model

dX1dt=X2

dX2dt=-X2+K*u1T+D

In the original MHE problem where the disturbance has to be estimated, D is assumed to be as a state with the following dynamics

dDdt=dX3dt=0

The code for MHE is modified to obtain parameter estimation, this is done by providing a constant value to the disturbance ‘D’ and rearranging the parameter ‘K’ as a state and providing the following dynamics.

dKdt=dX3dt=0

The model is transformed to the following form

dX1dt=X2

dX2dt=-X2+X3*u1T+D

dX3dt=0

The code is provided in Appendix and is also saved as the file with the name ‘Problem_2.m’.

The result from the parameter estimation using MHE is presented in the Figure 7. The plot shows that there is a close match between the values from the model and the real values, which infers that the estimated parameters are correct.

Figure  7. Parameter estimation using MHE

Problem 3

d

PROCESS

MPC

e

Ym

SENSOR

SP

y

u

Observer

dest

dini

Figure  10. Control block diagram for MPC

- The script works and the following plot is generated at the end of the run.

Figure  11. Control using MPC

- Done!

- Effect of changing the MPC cost function weights . C_du and C_e

| S.No | C_du | C_e | IAE | Figure |
| --- | --- | --- | --- | --- |
| 1 | 20 | 1 | 18.9663 | Figure |
| 2 | 15 | 1 | 18.9652 | Figure |
| 3 | 10 | 1 | 18.9622 | Figure |
| 4 | 5 | 1 | 18.9563 | Figure |
| 5 | 1 | 1 | 18.9114 | Figure |
| 6 | 0.1 | 1 | 19.0100 | Figure |
| 7 | 1 | 2 | 18.8895 | Figure |
| 8 | 1 | 5 | 18.9501 | Figure |

If from the figures presented in the section below, it can be observed that decreasing the C_du (cost function weights for changes in control variable) would result in a more aggressive controller behavior. The set-point tracking appears to be better in some case, but beyond a certain value, the more aggressive behavior doesn’t result in a better IAE.

- Effect of changing the prediction Horizon

| S.No | N_predict | IAE | Simulation time total/loop | Figure |
| --- | --- | --- | --- | --- |
| 1 | 8 | 18.9664 | 173.4603/ 0.2975 | Figure |
| 2 | 10 | 19.3217 | 206.9854/ 0.3575 | Figure |
| 3 | 12 | 19.6899 | 253.7028/ 0.4412 | Figure |
| 4 | 14 | 20.0045 | 254.1775/ 0.4451 | Figure |

Increasing the prediction horizon would increase the IAE, but by looking at the figures, it is observed that increasing the prediction horizon provides a better set point tracking. However, a higher simulation time implies that it take more time to calculate the control move.

- Introducing model error in the Gain

A constant error in the value of gain (Kp) is introduced by  replacing the variable ‘gain’ in the code with ‘gain+error’. The error of 0.5 is added to the gain and the simulator is run again. The results of the simulations are presented in Figure 12.

Figure  12. Control using MPC

It is observed from the figure above that the MPC can provide set point tracking also in the presence of a model error. Despite model error the observer can estimate the T_out. However, there is a constant offset between the real value of the disturbance (d_sim) and the value estimated by the observer (d_est). This is due to the fact that the observer includes the model error (in the ‘gain’ Kv) as a part of the total disturbance.

Appendix

- Problem 1a

clear all; clc;close all;

Ts=0.1; % timestep for integration

sim_time=100; %total simulation time

%% Model Parameters

A=0.1; %area of Level tank

Kp=0.002; %

%% Kalman Filter Parameters

Q=[0.01 0;0 0.0001];

R=0.000001;

P_initial=eye(2);

%% Initial condition

h(1)=0.5;

F_out(1)=0.004;

X_init=h;

u=2*ones(1,sim_time/Ts);

SP=0.5*ones(1,sim_time/Ts);

d=0.004*ones(1,sim_time/Ts);

% PID Controller parameters

Kc=10;

Ti=10;

cho=menu('Choose a case','Step in OutFlow','Step in Set-point');

if cho==1

t = Ts:Ts:sim_time;

step = t>=50;

S_S=input('Step size for the change in out flow (-0.004 to 0.004) ');

d=d+step*S_S;

end

if cho==2

S_S=input('Step size for the change in setpoint (-0.5 to 0.5) ');

t = Ts:Ts:sim_time;

step = t>=50;

SP=SP+step*S_S;

end

% Defining the system

A_cont=[0 -1/A; 0 0];

B_cont=[Kp/A;0];

C_cont=[1 0];

D_cont=[0];

sys_cont=ss(A_cont,B_cont,C_cont,D_cont);

sys_dis = c2d(sys_cont,Ts);

A_dis=sys_dis.a;

B_dis=sys_dis.b;

C_dis=sys_dis.c;

y=model_est(Ts,sim_time, Kp, A, Kc, Ti, SP, u, d, X_init, A_dis, B_dis,C_dis, Q, R, P_initial);

K=y(3:4,end)

%% Plot the results

subplot(3,1,1)

plot(y(1,:),'r');

hold on

plot(SP,'--');

hold on

plot(y(5,:),'-b');

xlabel('Time (s)');

ylabel('Level (m)');

legend('Estimated Level','Set Point','Level');

axis([0 sim_time/Ts 0 1]);

subplot(3,1,2)

plot(d);

hold on

plot(y(2,:),'-r');

xlabel('Time (s)');

ylabel('Flow (m3/s)');

legend('F_o_u_t','F_o_u_t_e_s_t');

axis([0 sim_time/Ts 0 0.01])

subplot(3,1,3)

plot(y(6,:));

xlabel('Time (s)');

ylabel('Control Signal');

axis([0 sim_time/Ts 0 6])

function y=model_est(Ts,sim_time, Kp,A,Kc,Ti,SP,u,d,X_init,A_dis,B_dis,C_dis,Q,R,P_initial)

n_time=sim_time/Ts;

X(1)=X_init;

Xest(:,1)=[X_init;d(1)];

h_meas(1)=X(1);

int=0;

P=P_initial;

for i=2:n_time

% Process Update------------------------

X(i)=(Kp/A*u(i-1)-1/A*d(i-1))*Ts+X(i-1);

h(i)=X(i);

%---------------------------------------

% Measurement Noise---------------------

noise=0.005;

h_meas(i)=h(i)+noise*rand;

%---------------------------------------

% Controller Update---------------------

error=SP(i-1)-h_meas(i-1);

int=int+error*Ts;

u(i)=Kc*error+Kc/Ti*int;

%---------------------------------------

% Kalman Filter-----------------------------------------------------

% predict

Xest_ap=A_dis*Xest(:,i-1)+B_dis*u(i);

P=A_dis*P*A_dis'+Q;

% update

y=h_meas(i)-C_dis*Xest_ap;

S=R+C_dis*P*C_dis';

K(:,i)=P*C_dis'*inv(S);

Xest(:,i)=Xest_ap+K(:,i)*y;

P=(eye(2)-K(:,i)*C_dis)*P*(eye(2)-K(:,i)*C_dis)'+K(:,i)*R*K(:,i)';

%-------------------------------------------------------------------

lvl=Xest(1,:);

Fout=Xest(2,:);

end

y=[lvl;Fout;K;h;u];

end

- Problem 1b

clear all; clc;

b=func_model_symb; % original non-linear model

a=func_lin_model_symb; %linearising the model to obtain A, B, C and D

%% Non-linear model which can be used for parameter eatimation

function model_symb = func_model_symb

syms X1

syms X2

syms u

syms z

syms k

syms Kv

syms Kp

syms A

syms rho

syms g

syms G

%State Equation:

f_X1 = Kp/A*u-1/A*k*z*X2*(rho*g*X1/G);

f_X2 = 0;

model_symb.f_X1 = f_X1;

model_symb.f_X2 = f_X2;

end

%% Symbolic Linearisation of the non-linear model

function lin_model_symb = func_lin_model_symb

syms X1

syms X2

syms u

syms z

syms k

syms Kv

syms Kp

syms A

syms rho

syms g

syms G

%State Equation:

f_X1 = Kp/A*u-1/A*k*z*X2*(rho*g*X1/G);

f_X2 = 0;

A_cont_symb = jacobian([f_X1;f_X2],[X1;X2]);

B_cont_symb = jacobian([f_X1;f_X2],[u]);

C_cont_symb = [1 0];

D_cont_symb = [0];

lin_model_symb.A_cont_symb = A_cont_symb;

lin_model_symb.B_cont_symb = B_cont_symb;

lin_model_symb.C_cont_symb = C_cont_symb;

lin_model_symb.D_cont_symb = D_cont_symb;

end

- Problem 2

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

%Finn Aakre Haugen (Modified by Abhilash Nair on 22.04.2018)

%15 04 2018

%MHE with model with 2 state variables and 1 parameter as augmented state:

%dx1_dt = x2 + w1

%dx2/dt = (-x2 + K*u + d)/T + w2

%y = x1 + v

%where K = x3 is to be estimated.

%

%Note 1: Using matrix as optim variable.

%Note 2: Objective and constraints functions for fmincon are defined

%as local functions at the end of this script.

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

disp('-------------------------------');

disp('Moving Horizon Estimator for:')

disp('dx1_dt=x2; dx2/dt=(-x2+K*u)/T;');

disp('x1, x2, K are estimated.');

disp('y=x1 is measured.');

disp('');

%-------------------------------------------

close all

clear all

clc

format compact

commandwindow

%-------------------------------------------

%Model params:

dist = 1; %Disturbance

T = 2;%s Time constant

n = 2;%Number of state variables

model_params.d = dist;

model_params.T = T;%model_params is struct. K is field.

cov_process_disturb_w1 = .001;

cov_process_disturb_w2 = .001;

cov_process_disturb_w3 = .001;

cov_process_disturb_w = ...

diag([cov_process_disturb_w1,cov_process_disturb_w2,...

cov_process_disturb_w3]);

cov_meas_noise_v1 = .01;

cov_meas_noise_v = diag([cov_meas_noise_v1]);

%--------------------------

%Time settings:

Ts = 0.5;%s

t_start = 0;%s

t_stop = 40;%s

t_array = [t_start:Ts:(t_stop-Ts)];%Array for storage

N = length(t_array);

t_mhe = 5

N_mhe = floor(t_mhe/Ts)

number_optim_vars = n*N_mhe

%-----------------------------------

%Preallocation of arrays for storage:

u_sim_array = t_array*0;

x1_sim_array = t_array*0;

x2_sim_array = t_array*0;

y1_sim_array = t_array*0;

x1_est_optim_plot_array = t_array*0;

x2_est_optim_plot_array = t_array*0;

%-----------------------------------

%Sim initialization:

x1_sim_init = 2;

x2_sim_init = 3;

x1_sim_k = x1_sim_init;

x2_sim_k = x2_sim_init;

%-----------------------------------

%MHE initialization:

mhe_array = zeros(1,N_mhe);

x1_est_init_guess = 0;

x2_est_init_guess = 0;

K_est_init_guess = 2;

x1_est_optim_array = zeros(1,N_mhe) + x1_est_init_guess;

x2_est_optim_array = zeros(1,N_mhe) + x2_est_init_guess;

K_est_optim_array = zeros(1,N_mhe) + K_est_init_guess;

x_est_guess_matrix = ...

[x1_est_optim_array;x2_est_optim_array;K_est_optim_array];

u_mhe_array = mhe_array*0;

y1_meas_mhe_array = mhe_array*0;

%-----------------------------------

%Figure size etc.:

fig_posleft=8;fig_posbottom=2;fig_width=24;fig_height=18;

fig_pos_size_1=[fig_posleft,fig_posbottom,fig_width,fig_height];

h = figure(1);

set(gcf,'Units','centimeters','Position',fig_pos_size_1);

figtext='Moving Horizon Estimator';

set(gcf,'Name',figtext,'NumberTitle','on')

%-----------------------------------

%Sim loop:

for k = 1:N

t_k = k*Ts;

%-----------------------------

%Process simulator:

if t_k < 2

u_k = 2;

K = 1;

end

if t_k >= 2 %Change of u

u_k = 2;

end

if t_k >= 8 %Change of d

K = 1;

end

if t_k >= 10 %Change of u

u_k = 6;

end

if t_k >= 15 %Change of d

K = 1.5;

end

if t_k >= 20 %Change of d

dist = 0.5;

end

%Derivatives:

dx1_sim_dt_k = x2_sim_k;

dx2_sim_dt_k = (-x2_sim_k + K*u_k + dist)/T;

f1_sim_k = x1_sim_k + Ts*dx1_sim_dt_k;

f2_sim_k = x2_sim_k + Ts*dx2_sim_dt_k;

%Integration and adding disturbance:

w1_sim_k = sqrt(cov_process_disturb_w1)*randn;

w2_sim_k = sqrt(cov_process_disturb_w2)*randn;

x1_sim_kp1 = f1_sim_k + w1_sim_k;

x2_sim_kp1 = f2_sim_k + w2_sim_k;

%Calculating output and adding meas noise:

v1_sim_k = sqrt(cov_meas_noise_v1)*randn;

y1_sim_k = x1_sim_k + v1_sim_k;

%Storage:

t_array(k) = t_k;

u_sim_array(k) = u_k;

x1_sim_array(k) = x1_sim_k;

x2_sim_array(k) = x2_sim_k;

K_sim_array(k) = K;

y1_sim_array(k) = y1_sim_k;

%Preparing for time shift:

x1_sim_k = x1_sim_kp1;

x2_sim_k = x2_sim_kp1;

%Updating u and y for use in MHE:

u_mhe_array = [u_mhe_array(2:N_mhe),u_k];

y1_meas_mhe_array = [y1_meas_mhe_array(2:N_mhe),y1_sim_k];

y_meas_mhe_array = [y1_meas_mhe_array];

%--------------------------------------------------------------------

if k > N_mhe

Q = cov_process_disturb_w;

R = cov_meas_noise_v;

covars.Q = Q;

covars.R = R;

%Matrices defining linear constraints for use in fmincon:

A_ineq = []; B_ineq = []; A_eq = []; B_eq = [];

%fmincon initialization:

x1_est_init_error = 0;

x2_est_init_error = 0;

K_est_init_error = 0;

x_est_init_error=[x1_est_init_error;x2_est_init_error;K_est_init_error];

%Guessed optim states:

%Guessed present state (x_k) is needed to calculate optimal present meas

%(y_k). Model is used in prediction:

x1_km1 = x1_est_optim_array(N_mhe);

x2_km1 = x2_est_optim_array(N_mhe);

K_km1 = K_est_optim_array(N_mhe);

dx1_dt_km1 = x2_km1;

dx2_dt_km1 = (-x2_km1 + K_km1*u_k + dist)/T;

dK_dt_km1 = 0;

x1_pred_k = x1_km1 + Ts*dx1_dt_km1;

x2_pred_k = x2_km1 + Ts*dx2_dt_km1;

K_pred_k = K_km1 + Ts*dK_dt_km1;

%Now, guessed optimal states are:

x1_est_guess_array = ...

[x1_est_optim_array(2:N_mhe),x1_pred_k];

x2_est_guess_array = ...

[x2_est_optim_array(2:N_mhe),x2_pred_k];

K_est_guess_array = ...

[K_est_optim_array(2:N_mhe), K_pred_k];

x_est_guess_matrix = ...

[x1_est_guess_array;x2_est_guess_array;K_est_guess_array];

%Lower and upper limits of optim variables:

x1_est_max = 300;

x2_est_max = 20;

K_est_max = 5;

x1_est_max_array = zeros(1,N_mhe) + x1_est_max;

x2_est_max_array = zeros(1,N_mhe) + x2_est_max;

K_est_max_array = zeros(1,N_mhe) + K_est_max;

x1_est_min = -300;

x2_est_min = -00;

K_est_min = -5;

x1_est_min_array = zeros(1,N_mhe) + x1_est_min;

x2_est_min_array = zeros(1,N_mhe) + x2_est_min;

K_est_min_array = zeros(1,N_mhe) + K_est_min;

x_est_ub_matrix = [x1_est_max_array;x2_est_max_array;K_est_max_array];

x_est_lb_matrix = [x1_est_min_array;x2_est_min_array; K_est_min_array];

%Creating function handles:

fun_objective_handle = ...

@(x_est_matrix) fun_objective_mhe(x_est_matrix,...

y_meas_mhe_array,u_mhe_array,model_params,covars,x_est_init_error,n,N_mhe,Ts);

fun_constraints_handle = ...

@(x_est_matrix) fun_constraints_mhe(x_est_matrix,...

y_meas_mhe_array,u_mhe_array,model_params,covars,x_est_init_error,n,N_mhe,Ts);

%Calculating MHE estimate using fmincon:

%fmincon_options = optimoptions(@fmincon);

fmincon_options = optimoptions(@fmincon,'display','none');

%     fmincon_options = optimoptions(@fmincon,'algorithm','sqp','display','none');

[x_est_optim_matrix,fval,exitflag,output,lambda,grad,hessian] = ...

fmincon(fun_objective_handle,x_est_guess_matrix,A_ineq,...

B_ineq,A_eq,B_eq,x_est_lb_matrix,x_est_ub_matrix,...

fun_constraints_handle,fmincon_options);

x1_est_optim_array = x_est_optim_matrix(1,:);

x2_est_optim_array = x_est_optim_matrix(2,:);

K_est_optim_array = x_est_optim_matrix(3,:);

% fval

end %if

x1_est_optim_plot_array(k) = x1_est_optim_array(end);

x2_est_optim_plot_array(k) = x2_est_optim_array(end);

K_est_optim_plot_array(k) = K_est_optim_array(end);

%Continuous plotting:

x_lim_array=[t_start,t_stop];

if (k>1 & k<N)

if k < N_mhe

pause(1);

else

pause(0);

end

subplot(4,1,1)

plot([t_array(k-1),t_array(k)],...

[u_sim_array(k-1),u_sim_array(k)],'b-o');

if k==2

hold on

grid minor

xlim(x_lim_array);

ylim([0 10]);

title('u')

%ylabel('[m]')

%xlabel('t [s]')

end

subplot(4,1,2)

plot([t_array(k-1),t_array(k)],...

[x1_est_optim_plot_array(k-1),x1_est_optim_plot_array(k)],'r-o',...

[t_array(k-1),t_array(k)],...

[x1_sim_array(k-1),x1_sim_array(k)],'b-o');

if k==2

hold on

grid minor

xlim(x_lim_array);

ylim([0 300]);

title('x1\_sim = blue. x1\_mhe\_est = red.')

%ylabel('[m]')

%xlabel('t [s]')

end

subplot(4,1,3)

plot([t_array(k-1),t_array(k)],...

[x2_est_optim_plot_array(k-1),x2_est_optim_plot_array(k)],'r-o',...

[t_array(k-1),t_array(k)],...

[x2_sim_array(k-1),x2_sim_array(k)],'b-o');

if k==2

hold on

grid minor

xlim(x_lim_array);

ylim([0 20]);

title('x2\_sim = blue. x2\_mhe\_est = red.')

%ylabel('[m]')

%xlabel('t [s]')

end

subplot(4,1,4)

plot([t_array(k-1),t_array(k)],...

[K_est_optim_plot_array(k-1),K_est_optim_plot_array(k)],'r-o',...

[t_array(k-1),t_array(k)],...

[K_sim_array(k-1),K_sim_array(k)],'b-o');

if k==2

hold on

grid minor

xlim(x_lim_array);

ylim([0 3]);

title('K\_real = blue. K\_mhe\_est = red.')

%ylabel('[m]')

xlabel('t [s]')

end

end %if (k>1 & k<N)

end %sim loop

%----------------------------------------------------

%Printing figure as pdf file:

saveas(h,'example_mhe','pdf')

%----------------------------------------------------

%Defining local functions:

%----------------------------------------------------

function f = fun_objective_mhe(x_est_matrix,...

y_meas_mhe_array,u_mhe_array,model_params,covars,x_est_init_error,n,N_mhe,Ts)

dist = model_params.d;

T = model_params.T;

Q = covars.Q;

R = covars.R;

J_km1 = 0;

y1_meas_mhe_array = y_meas_mhe_array(1,:);

for k = 1:N_mhe

u_k = u_mhe_array(1,k);

x_k = x_est_matrix(:,k);

x1_k = x_k(1);

x2_k = x_k(2);

K = x_k(3);

h1_k = x1_k;

y1_meas_k = y1_meas_mhe_array(1,k);

v1_k = y1_meas_k - h1_k;

v_k = [v1_k];

if k <= N_mhe-1

x_kp1 = x_est_matrix(:,k+1);

x1_kp1 = x_kp1(1);

x2_kp1 = x_kp1(2);

K_kp1 = x_kp1(3);

dx1_dt_k = x2_k;

dx2_dt_k = (-x2_k + K_kp1*u_k + dist)/T;

dK_dt_k = 0;

f1_k = x1_k + Ts*dx1_dt_k;

f2_k = x2_k + Ts*dx2_dt_k;

f3_k = K + Ts*dK_dt_k;

w1_k = x1_kp1 - f1_k;

w2_k = x2_kp1 - f2_k;

w3_k = K_kp1 - f3_k;

w_k = [w1_k,w2_k,w3_k]';

end

dJ_k = w_k'*inv(Q)*w_k + v_k'*inv(R)*v_k;

J_k = J_km1 + dJ_k;

%Time shift:

J_km1 = J_k;

end %for k=1:N_mhe

f = J_k;

end

%------------------------------------------------

function [cineq,ceq]=fun_constraints_mhe(x_est_matrix,...

y_meas_mhe_array,u_mhe_array,model_params,covars,x_est_init_error,n,N,Ts)

cineq = [];   % Compute nonlinear inequalities. Calculated below.

ceq = [];   % Compute nonlinear equalities.

end

%------------------------------------------------

- Problem 3

%Finn Haugen, USN

%16 April 2018

%MPC control of simulated air heater

%http://home.usn.no/finnh/air_heater/

%----------------------------------

clear all

format short

close all

%----------------------------------

%Process params:

gain = 3.5; %[deg C]/[V]

theta_const = 23; %[s]

theta_delay = 3; %[s]

model_params.gain = gain;

model_params.theta_const = theta_const;

model_params.theta_delay = theta_delay;

Temp_env_k = 25; %[deg C]

%----------------------------------

%Time settings:

Ts = 0.5; %Time-step [s]

t_pred_horizon = 8;

N_pred = t_pred_horizon/Ts;

t_start = 0;

t_stop = 300;

N_sim = (t_stop-t_start)/Ts;

t = [t_start:Ts:t_stop-Ts];

%-----------------------

%MPC costs:

C_e = 5;

C_du = 1;

mpc_costs.C_e = C_e;

mpc_costs.C_du = C_du;

%----------------------------------

%Defining sequence for temp_out setpoint:

Temp_sp_const = 30; %[C]

Ampl_step = 2; %[C]

Slope = -0.04; %[C/s]

Ampl_sine = 1; %[C]

T_period = 50; %[s]

t_const_start = t_start;t_const_stop = 100;

t_step_start = t_const_stop;t_step_stop = 150;

t_ramp_start = t_step_stop;t_ramp_stop = 200;

t_sine_start = t_ramp_stop;t_sine_stop = 250;

t_const2_start = t_sine_stop;t_const2_stop = t_stop;

for k = 1:N_sim

if (t(k) >= t_const_start & t(k) < t_const_stop),

Temp_sp_array(k) = Temp_sp_const;

end

if (t(k) >= t_step_start & t(k) < t_step_stop),

Temp_sp_array(k)=Temp_sp_const + Ampl_step;

end

if (t(k) >= t_ramp_start & t(k) < t_ramp_stop),

Temp_sp_array(k) = Temp_sp_const+Ampl_step+Slope*(t(k)-t_ramp_start);

end

if (t(k) >= t_sine_start & t(k) < t_sine_stop),

Temp_sp_array(k) = ...

Temp_sp_const+Ampl_sine*sin(2*pi*(1/T_period)*(t(k)-t_sine_start));

end

if (t(k) >= t_const2_start),

Temp_sp_array(k) = Temp_sp_const;

end

end

%----------------------------------

%Initialization:

u_init = 0;

N_delay = floor(theta_delay/Ts) + 1;

delay_array = zeros(1,N_delay) + u_init;

%----------------------------------

%Initial guessed optimal control sequence:

Temp_heat_sim_k = 0; %[C]

Temp_out_sim_k = 28; %[C]

d_sim_k = -0.5;

%----------------------------------

%Initial values for estimator:

Temp_heat_est_k = 0; %[C]

Temp_out_est_k = 25; %[C]

d_est_k = 0; %[V]

%----------------------------------

%Initial guessed optimal control sequence:

u_guess = zeros(N_pred,1) + u_init;

%----------------------------------

%Initial value of previous optimal value:

u_opt_km1 = u_init;

%----------------------------------

%Defining arrays for plotting:

t_plot_array = zeros(1,N_sim);

Temp_out_sp_plot_array = zeros(1,N_sim);

Temp_out_sim_plot_array = zeros(1,N_sim);

u_plot_array = zeros(1,N_sim);

d_est_plot_array = zeros(1,N_sim);

d_sim_plot_array = zeros(1,N_sim);

%----------------------------------

%Matrices defining linear constraints for use in fmincon:

A = [];

B = [];

Aeq = [];

Beq = [];

%----------------------------------

%Lower and upper limits of optim variable for use in fmincon:

u_max = 5;

u_min = 0;

u_ub = zeros(1,N_pred) + u_max;

u_lb = zeros(1,N_pred) + u_min;

u_delayed_k = 2;

%------------------------

%Calculation of observer gain for estimation of input disturbance, d:

A_est = [-1/theta_const, gain/theta_const; 0,0];

C_est = [1 0];

n_est = length(A_est);

Tr_est = 5;

T_est = Tr_est/n_est;

estimpoly = [T_est*T_est,sqrt(2)*T_est,1];

eig_est = roots(estimpoly);

K_est1 = acker(A_est',C_est',eig_est);

K_est = K_est1';

%-----------------------------------

%Figure settings:

fig_posleft=8;fig_posbottom=2;fig_width=24;fig_height=18;

fig_pos_size_1=[fig_posleft,fig_posbottom,fig_width,fig_height];

h = figure(1);

set(gcf,'Units','centimeters','Position',fig_pos_size_1);

figtext='MPC control of air heater';

set(gcf,'Name',figtext,'NumberTitle','on')

%----------------------------------

%For-loop for MPC of simulated process incl Kalman Filter:

tic

for k = 1:(N_sim-N_pred)

t_k = t(k);

t_plot_array(k)= t_k;

%-----------------------

%Observer for estimating input-disturbance d using Temp_out:

%Note: The time-delayed u is used as control signal shere.

e_est_k = Temp_out_sim_k - Temp_out_est_k;

dTemp_heat_est_dt_k = ...

(1/theta_const)*(-Temp_heat_est_k + gain*(u_delayed_k + d_est_k))...

+ K_est(1)*e_est_k;

dd_est_dt_k = 0 + K_est(2)*e_est_k;

Temp_heat_est_kp1 = Temp_heat_est_k + Ts*dTemp_heat_est_dt_k;

d_est_kp1 = d_est_k + Ts*dd_est_dt_k;

Temp_out_est_k = Temp_heat_est_k + Temp_env_k;

%-----------------------

%Storage for plotting:

Temp_out_est_plot_array(k) = Temp_out_est_k;

d_est_plot_array(k) = d_est_k;

%-----------------------

%Setpoint array to optimizer:

Temp_sp_to_mpc_array = Temp_sp_array(k:k+N_pred);

Temp_out_sp_plot_array(k) = Temp_sp_array(k);

%-----------------------

%Estimated state to optimizer:

state_est.Temp_heat_est_k = Temp_heat_est_k;

state_est.d_est_k = d_est_k;

%-----------------------

%Calculating optimal control sequence:

fun_handle = @(u) fun_objectfunction_mpc_airheater...

(u,u_opt_km1,state_est,Temp_env_k,...

Temp_sp_to_mpc_array,model_params,mpc_costs,N_pred,Ts);

fmincon_options = optimoptions(@fmincon,'display','none');

%     fmincon_options = optimoptions(@fmincon,'algorithm','active-set','display','none');

%     fmincon_options = optimoptions(@fmincon,'algorithm','sqp','display','none');

[u_opt,fval,exitflag,output,lambda,grad,hessian] =...

fmincon(fun_handle,u_guess,A,B,Aeq,Beq,u_lb,u_ub,@fun_constraints_mpc_airheater,fmincon_options);

u_guess = u_opt; %Optimal solution to be used as guessed solution in next iteration.

u_k = u_opt(1); %Optimal control signal (sample) to be applied

u_plot_array(k) = u_k; %Storage for plotting

u_opt_km1 = u_opt(1);

%------------------------------

%Applying optimal control signal to simulated process:

d_sim_k = -0.5;

d_sim_plot_array(k) = d_sim_k;

u_delayed_k = delay_array(N_delay);

u_nondelayed_k = u_k;

delay_array = [u_nondelayed_k,delay_array(1:end-1)];

dTemp_heat_sim_dt_k = ...

(1/theta_const)*(-Temp_heat_sim_k + gain*(u_delayed_k + d_sim_k));

Temp_heat_sim_kp1 = Temp_heat_sim_k + Ts*dTemp_heat_sim_dt_k;

Temp_out_sim_k = Temp_heat_sim_k + Temp_env_k;

Temp_out_sim_plot_array(k) = Temp_out_sim_k;%Storage for plotting

%------------------------------

%Time shift for estimator and for simulator:

Temp_heat_est_k = Temp_heat_est_kp1;

d_est_k = d_est_kp1;

Temp_heat_sim_k = Temp_heat_sim_kp1;

%------------------------------

%Continuous plotting:

x_lim_array = [t_start,t_stop];

if (k>1 & k<N_sim)

pause(0.0);

subplot(3,1,1)

plot([t_plot_array(k-1),t_plot_array(k)],...

[Temp_out_sp_plot_array(k-1),Temp_out_sp_plot_array(k)],'r-',...

[t_plot_array(k-1),t_plot_array(k)],...

[Temp_out_sim_plot_array(k-1),Temp_out_sim_plot_array(k)],'b-',...

[t_plot_array(k-1),t_plot_array(k)],...

[Temp_out_est_plot_array(k-1),Temp_out_est_plot_array(k)],'m-');

if k==2

hold on

grid minor

xlim(x_lim_array);

ylim([28 33]);

title('Temp\_out\_sp = red. Temp\_out\_sim = blue. Temp\_out\_est = magenta.')

ylabel('[deg C]')

xlabel('t [s]')

end

subplot(3,1,2)

plot([t_plot_array(k-1),t_plot_array(k)],...

[u_plot_array(k-1),u_plot_array(k)],'b-');

if k==2

hold on

grid minor

xlim(x_lim_array);

ylim([0 5]);

title('Control signal u')

ylabel('[V]')

xlabel('t [s]')

end

subplot(3,1,3)

plot([t_plot_array(k-1),t_plot_array(k)],...

[d_est_plot_array(k-1),d_est_plot_array(k)],'r-',...

[t_plot_array(k-1),t_plot_array(k)],...

[d_sim_plot_array(k-1),d_sim_plot_array(k)],'b-');

if k==2

hold on

grid minor

xlim(x_lim_array);

%             ylim([0 10]);

title('d\_est = red. d\_sim = blue.')

%ylabel('[m]')

xlabel('t [s]')

end

end %if (k>1 & k<N)

end

IAE(num)=norm(Temp_out_sp_plot_array-Temp_out_sim_plot_array);

%Elapsed total time and avg loop time:

toc_total=toc

toc_loop=toc_total/k

%----------------------------------------------------

%Printing figure as pdf file:

% saveas(h,'example_mpc_air_heater','pdf')

%----------------------------------------------------

%Functions defined as local functions:

function f = fun_objectfunction_mpc_airheater(u,u_opt_km1,state_est,Temp_env_k,...

Temp_sp_to_mpc_array,model_params,mpc_costs,N_pred,Ts)

gain = model_params.gain;

theta_const = model_params.theta_const;

theta_delay = model_params.theta_delay;

C_e = mpc_costs.C_e;

C_du = mpc_costs.C_du;

Temp_heat_k = state_est.Temp_heat_est_k;

d_k = state_est.d_est_k;

N_delay = floor(theta_delay/Ts) + 1;

delay_array = zeros(1,N_delay) + u(1);

% ru_km1 = u_opt_km1;

u_km1 = u(1);

J_km1 = 0;

%Applying optimal control signal to simulated process using explicit Euler:

for k = 1:N_pred

u_k = u(k);

Temp_sp_k = Temp_sp_to_mpc_array(k);

%Time delay:

u_delayed_k = delay_array(N_delay);

u_nondelayed_k = u_k;

delay_array = [u_nondelayed_k,delay_array(1:end-1)];

%Solving diff eq:

dTemp_heat_dt_k = ...

(1/theta_const)*(-Temp_heat_k + gain*(u_delayed_k + d_k));

Temp_heat_kp1 = Temp_heat_k + Ts*dTemp_heat_dt_k;

Temp_out_k = Temp_heat_k + Temp_env_k;

%Updating objective function:

e_k = Temp_sp_k - Temp_out_k;

du_k = (u_k - u_km1)/Ts;

J_k = J_km1 + Ts*(C_e*e_k^2 + C_du*du_k^2);

%Time shift:

Temp_heat_k = Temp_heat_kp1;

u_km1 = u_k;

J_km1 = J_k;

end

f = J_k;

end

%--------------------------------

function [cineq,ceq]=fun_constraints_mpc_airheater(u,u_opt_km1,state_est,Temp_env_k,...

Temp_sp_to_mpc_array,model_params,mpc_costs,N_pred,Ts)

cineq =  [];   % Compute nonlinear inequalities.

ceq = [];   % Compute nonlinear equalities.

end

- Problem 3e

%Finn Haugen, USN

%16 April 2018

%MPC control of simulated air heater

%http://home.usn.no/finnh/air_heater/

%----------------------------------

clear all

format short

close all

%----------------------------------

%Process params:

gain = 3.5; %[deg C]/[V]

theta_const = 23; %[s]

theta_delay = 3; %[s]

err=0.4;

model_params.gain = gain;

model_params.theta_const = theta_const;

model_params.theta_delay = theta_delay;

Temp_env_k = 25; %[deg C]

%----------------------------------

%Time settings:

Ts = 0.5; %Time-step [s]

t_pred_horizon = 8;

N_pred = t_pred_horizon/Ts;

t_start = 0.5;

t_stop = 300;

N_sim = (t_stop-t_start)/Ts;

t = [t_start:Ts:t_stop-Ts];

%-----------------------

%MPC costs:

C_e = 1;

C_du = 20;

mpc_costs.C_e = C_e;

mpc_costs.C_du = C_du;

%----------------------------------

%Defining sequence for temp_out setpoint:

Temp_sp_const = 30; %[C]

Ampl_step = 2; %[C]

Slope = -0.04; %[C/s]

Ampl_sine = 1; %[C]

T_period = 50; %[s]

t_const_start = t_start;t_const_stop = 100;

t_step_start = t_const_stop;t_step_stop = 150;

t_ramp_start = t_step_stop;t_ramp_stop = 200;

t_sine_start = t_ramp_stop;t_sine_stop = 250;

t_const2_start = t_sine_stop;t_const2_stop = t_stop;

for k = 1:N_sim

if (t(k) >= t_const_start & t(k) < t_const_stop),

Temp_sp_array(k) = Temp_sp_const;

end

if (t(k) >= t_step_start & t(k) < t_step_stop),

Temp_sp_array(k)=Temp_sp_const + Ampl_step;

end

if (t(k) >= t_ramp_start & t(k) < t_ramp_stop),

Temp_sp_array(k) = Temp_sp_const%+Ampl_step+Slope*(t(k)-t_ramp_start);

end

if (t(k) >= t_sine_start & t(k) < t_sine_stop),

Temp_sp_array(k) = ...

Temp_sp_const%+Ampl_sine*sin(2*pi*(1/T_period)*(t(k)-t_sine_start));

end

if (t(k) >= t_const2_start),

Temp_sp_array(k) = Temp_sp_const;

end

end

%----------------------------------

%Initialization:

u_init = 0;

N_delay = floor(theta_delay/Ts) + 1;

delay_array = zeros(1,N_delay) + u_init;

%----------------------------------

%Initial guessed optimal control sequence:

Temp_heat_sim_k = 0; %[C]

Temp_out_sim_k = 28; %[C]

d_sim_k = -0.5;

%----------------------------------

%Initial values for estimator:

Temp_heat_est_k = 0; %[C]

Temp_out_est_k = 25; %[C]

d_est_k = 0; %[V]

%----------------------------------

%Initial guessed optimal control sequence:

u_guess = zeros(N_pred,1) + u_init;

%----------------------------------

%Initial value of previous optimal value:

u_opt_km1 = u_init;

%----------------------------------

%Defining arrays for plotting:

t_plot_array = zeros(1,N_sim);

Temp_out_sp_plot_array = zeros(1,N_sim);

Temp_out_sim_plot_array = zeros(1,N_sim);

u_plot_array = zeros(1,N_sim);

d_est_plot_array = zeros(1,N_sim);

d_sim_plot_array = zeros(1,N_sim);

%----------------------------------

%Matrices defining linear constraints for use in fmincon:

A = [];

B = [];

Aeq = [];

Beq = [];

%----------------------------------

%Lower and upper limits of optim variable for use in fmincon:

u_max = 5;

u_min = 0;

u_ub = zeros(1,N_pred) + u_max;

u_lb = zeros(1,N_pred) + u_min;

u_delayed_k = 2;

%------------------------

%Calculation of observer gain for estimation of input disturbance, d:

A_est = [-1/theta_const, gain/theta_const; 0,0];

C_est = [1 0];

n_est = length(A_est);

Tr_est = 5;

T_est = Tr_est/n_est;

estimpoly = [T_est*T_est,sqrt(2)*T_est,1];

eig_est = roots(estimpoly);

K_est1 = acker(A_est',C_est',eig_est);

K_est = K_est1'

%-----------------------------------

%Figure settings:

fig_posleft=8;fig_posbottom=2;fig_width=24;fig_height=18;

fig_pos_size_1=[fig_posleft,fig_posbottom,fig_width,fig_height];

h = figure(1);

set(gcf,'Units','centimeters','Position',fig_pos_size_1);

figtext='MPC control of air heater';

set(gcf,'Name',figtext,'NumberTitle','on')

%----------------------------------

%For-loop for MPC of simulated process incl Kalman Filter:

tic

for k = 1:(N_sim-N_pred)

t_k = t(k);

t_plot_array(k)= t_k;

%-----------------------

%Observer for estimating input-disturbance d using Temp_out:

%Note: The time-delayed u is used as control signal shere.

e_est_k = Temp_out_sim_k - Temp_out_est_k;

dTemp_heat_est_dt_k = ...

(1/theta_const)*(-Temp_heat_est_k + (gain)*(u_delayed_k + d_est_k))...

+ K_est(1)*e_est_k;

dd_est_dt_k = 0 + K_est(2)*e_est_k;

Temp_heat_est_kp1 = Temp_heat_est_k + Ts*dTemp_heat_est_dt_k;

d_est_kp1 = d_est_k + Ts*dd_est_dt_k;

Temp_out_est_k = Temp_heat_est_k + Temp_env_k;

%-----------------------

%Storage for plotting:

Temp_out_est_plot_array(k) = Temp_out_est_k;

d_est_plot_array(k) = d_est_k;

%-----------------------

%Setpoint array to optimizer:

Temp_sp_to_mpc_array = Temp_sp_array(k:k+N_pred);

Temp_out_sp_plot_array(k) = Temp_sp_array(k);

%-----------------------

%Estimated state to optimizer:

state_est.Temp_heat_est_k = Temp_heat_est_k;

state_est.d_est_k = d_est_k;

%-----------------------

%Calculating optimal control sequence:

fun_handle = @(u) fun_objectfunction_mpc_airheater...

(u,u_opt_km1,state_est,Temp_env_k,...

Temp_sp_to_mpc_array,model_params,mpc_costs,N_pred,Ts);

fmincon_options = optimoptions(@fmincon,'display','none');

%     fmincon_options = optimoptions(@fmincon,'algorithm','active-set','display','none');

%     fmincon_options = optimoptions(@fmincon,'algorithm','sqp','display','none');

[u_opt,fval,exitflag,output,lambda,grad,hessian] =...

fmincon(fun_handle,u_guess,A,B,Aeq,Beq,u_lb,u_ub,@fun_constraints_mpc_airheater,fmincon_options);

u_guess = u_opt; %Optimal solution to be used as guessed solution in next iteration.

u_k = u_opt(1); %Optimal control signal (sample) to be applied

u_plot_array(k) = u_k; %Storage for plotting

u_opt_km1 = u_opt(1);

%------------------------------

%Applying optimal control signal to simulated process:

d_sim_k = -0.5;

d_sim_plot_array(k) = d_sim_k;

u_delayed_k = delay_array(N_delay);

u_nondelayed_k = u_k;

delay_array = [u_nondelayed_k,delay_array(1:end-1)];

dTemp_heat_sim_dt_k = ...

(1/theta_const)*(-Temp_heat_sim_k + (gain + err)*(u_delayed_k + d_sim_k));

Temp_heat_sim_kp1 = Temp_heat_sim_k + Ts*dTemp_heat_sim_dt_k;

Temp_out_sim_k = Temp_heat_sim_k + Temp_env_k;

Temp_out_sim_plot_array(k) = Temp_out_sim_k;%Storage for plotting

%------------------------------

%Time shift for estimator and for simulator:

Temp_heat_est_k = Temp_heat_est_kp1;

d_est_k = d_est_kp1;

Temp_heat_sim_k = Temp_heat_sim_kp1;

%------------------------------

%Continuous plotting:

x_lim_array = [t_start,t_stop];

if (k>1 & k<N_sim)

pause(0.0);

subplot(3,1,1)

plot([t_plot_array(k-1),t_plot_array(k)],...

[Temp_out_sp_plot_array(k-1),Temp_out_sp_plot_array(k)],'r-',...

[t_plot_array(k-1),t_plot_array(k)],...

[Temp_out_sim_plot_array(k-1),Temp_out_sim_plot_array(k)],'b-',...

[t_plot_array(k-1),t_plot_array(k)],...

[Temp_out_est_plot_array(k-1),Temp_out_est_plot_array(k)],'m-');

if k==2

hold on

grid minor

xlim(x_lim_array);

ylim([28 33]);

title('Temp\_out\_sp = red. Temp\_out\_sim = blue. Temp\_out\_est = magenta.')

ylabel('[deg C]')

xlabel('t [s]')

end

subplot(3,1,2)

plot([t_plot_array(k-1),t_plot_array(k)],...

[u_plot_array(k-1),u_plot_array(k)],'b-');

if k==2

hold on

grid minor

xlim(x_lim_array);

ylim([0 5]);

title('Control signal u')

ylabel('[V]')

xlabel('t [s]')

end

subplot(3,1,3)

plot([t_plot_array(k-1),t_plot_array(k)],...

[d_est_plot_array(k-1),d_est_plot_array(k)],'r-',...

[t_plot_array(k-1),t_plot_array(k)],...

[d_sim_plot_array(k-1),d_sim_plot_array(k)],'b-');

if k==2

hold on

grid minor

xlim(x_lim_array);

%             ylim([0 10]);

title('d\_est = red. d\_sim = blue.')

%ylabel('[m]')

xlabel('t [s]')

end

end %if (k>1 & k<N)

end

IAE=norm(Temp_out_sp_plot_array-Temp_out_sim_plot_array)

%Elapsed total time and avg loop time:

toc_total=toc

toc_loop=toc_total/k

%----------------------------------------------------

%Printing figure as pdf file:

% saveas(h,'example_mpc_air_heater','pdf')

%----------------------------------------------------

%Functions defined as local functions:

function f = fun_objectfunction_mpc_airheater(u,u_opt_km1,state_est,Temp_env_k,...

Temp_sp_to_mpc_array,model_params,mpc_costs,N_pred,Ts)

gain = model_params.gain;

theta_const = model_params.theta_const;

theta_delay = model_params.theta_delay;

C_e = mpc_costs.C_e;

C_du = mpc_costs.C_du;

Temp_heat_k = state_est.Temp_heat_est_k;

d_k = state_est.d_est_k;

N_delay = floor(theta_delay/Ts) + 1;

delay_array = zeros(1,N_delay) + u(1);

% ru_km1 = u_opt_km1;

u_km1 = u(1);

J_km1 = 0;

%Applying optimal control signal to simulated process using explicit Euler:

for k = 1:N_pred

u_k = u(k);

Temp_sp_k = Temp_sp_to_mpc_array(k);

%Time delay:

u_delayed_k = delay_array(N_delay);

u_nondelayed_k = u_k;

delay_array = [u_nondelayed_k,delay_array(1:end-1)];

%Solving diff eq:

dTemp_heat_dt_k = ...

(1/theta_const)*(-Temp_heat_k + gain*(u_delayed_k + d_k));

Temp_heat_kp1 = Temp_heat_k + Ts*dTemp_heat_dt_k;

Temp_out_k = Temp_heat_k + Temp_env_k;

%Updating objective function:

e_k = Temp_sp_k - Temp_out_k;

du_k = (u_k - u_km1)/Ts;

J_k = J_km1 + Ts*(C_e*e_k^2 + C_du*du_k^2);

%Time shift:

Temp_heat_k = Temp_heat_kp1;

u_km1 = u_k;

J_km1 = J_k;

end

f = J_k;

end

%--------------------------------

function [cineq,ceq]=fun_constraints_mpc_airheater(u,u_opt_km1,state_est,Temp_env_k,...

Temp_sp_to_mpc_array,model_params,mpc_costs,N_pred,Ts)

cineq =  [];   % Compute nonlinear inequalities.

ceq = [];   % Compute nonlinear equalities.

end